| CONSERVATION PRACTICE PHYSICAL EFFECTS WORKSHEET |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| STATE | Any | FIELD OFFICE |  | Any |  | DATE | 7/31/2026 |
| PRACTICE: Deep Tillage 324 |  |  | Baseline Setting: |  |  |  |  |
|  |  |  | Appropriate Land Use(s): Crop |  |  |  |  |
| RESOURCES, CONSIDERATIONS AND CONCERNS |  |  | PHYSICAL EFFECTS |  | RATIONALE |  |  |
| SOIL - EROSION |  |  |  |  |  |  |  |
| Sheet and Rill |  |  | Neutral |  | Removing restrictive layers improves infiltration and reduces runoff. Deep Tillage loosens the soil and buries protective residue and live cover making it more prone to water erosion. |  |  |
| Wind |  |  | Slight to Moderate Improvement |  | If done at critical wind periods, deep tillage may temporarily increase surface roughness reducing saltation. It may also bring non-erodible materials to the surface and reduce wind erosion |  |  |
| Ephemeral Gully |  |  | Neutral |  | Removing restrictive layers improves infiltration and reduces runoff. Deep tillage refills the voided concentrated flow channels with loose soil where it is prone to being eroded away by the next event. |  |  |
| Classic Gully |  |  | Not Applicable |  | Not Applicable |  |  |
| Streambank |  |  | Not Applicable |  | Not Applicable |  |  |
| Shoreline |  |  | Not Applicable |  | Not applicable. |  |  |
| Irrigation Induced |  |  | Slight Improvement |  | Improves infiltration and reduces runoff. |  |  |
| Mass Movement |  |  | Slight Worsening |  | Increased infiltration could exacerbate mass movement during high rainfall. |  |  |
| Road, Roadsides, and Construction Sites |  |  | Slight to Moderate Improvement |  | Improves infiltration and reduces runoff and removes compaction to prepare sites for re-vegetation. |  |  |
| SOIL – CONDITION |  |  |  |  |  |  |  |
| Organic Matter Depletion |  |  | Moderate to Substantial Worsening |  | Deep tillage can bury the organic horizon and reduce the organic matter content by diluting it with subsoil materials. The mixing and inversion action of deep moldboard plows, or the lifting and fracturing of deep rippers increases the oxidation of organic matter. |  |  |
| Rangeland Site Stability |  |  | Not Applicable |  | Not applicable. |  |  |
| Compaction |  |  | Substantial Improvement |  | Ripping breaks up compaction, improves plant soil moisture, promotes root growth, and soil structure. |  |  |
| Subsidence |  |  | Slight Worsening |  | Deep tillage causes mixing and aeration. Histosols are subject to subsidence if drained and tilled. Deep tillage will increase oxidation of organic soils. |  |  |
| Contaminants: |  |  |  |  |  |  |  |
| - Salts and other Chemicals |  |  | Slight to Moderate Improvement |  | Improved infiltration and porosity leaches salts. Deep plowing will bury and dilute the contaminant. |  |  |
| - Animal Waste and other Organics - N |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Animal Waste and other Organics - P |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Animal Waste and other Organics - K |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Commercial Fertilizer - N |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Commercial Fertilizer – P |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Commercial Fertilizer – K |  |  | Slight to Moderate Improvement |  | Ripping improves infiltration which increases leaching of mineralized nutrients. Deep plowing will bury and dilute the contaminant. |  |  |
| - Residual Pesticides |  |  | Slight to Moderate Improvement |  | Ripping mixes the soil and can result in adsorption and deactivation. Deep plowing will bury and dilute the contaminant. |  |  |
| Damage from Sediment Deposition |  |  | Slight to Moderate Improvement |  | Deep plowing and ripping buries or mixes undesirable soil deposits from wind or water erosion or flood overwash. |  |  |
| - WATER – QUANTITY |  |  |  |  |  |  |  |
| - Rangeland Hydrologic Cycle |  |  | Not Applicable |  | Not applicable. |  |  |
| - Excessive Seepage |  |  | Slight to Moderate Worsening |  | Deep tillage may temporarily increase soil water content. |  |  |
| - Excessive Runoff, Flooding, or Ponding |  |  | Not Applicable |  | Not Applicable |  |  |
| - Excessive Subsurface Water |  |  | Slight to Moderate Improvement |  | Deep ripping a tillage pan or fragipan can remove a perched water table in the root zone. |  |  |
| - Drifted Snow |  |  | Not Applicable |  | Not applicable. |  |  |
| - Inadequate Outlets |  |  | Not Applicable |  | Not Applicable |  |  |
| - Inefficient Water use on Irrigated Land |  |  | Slight to Moderate Improvement |  | Increases infiltration and reduces runoff. |  |  |
| Inefficient Water use on Non-Irrigated Land |  |  | Slight to Moderate Improvement |  | - Deep tillage increases infiltration and reduces runoff. |  |  |
| Reduced Capacity of Conveyances by Sediment Deposition |  |  | Neutral |  | - Increase in infiltration is negated by the increased erosion of the bare and loosened soil conditions that result from deep tillage. |  |  |
| - Reduced Storage of Water Bodies by Sediment Accumulation |  |  | Neutral |  | - Increase in infiltration is negated by the increased erosion of the bare and loosened soil conditions that result from deep tillage. |  |  |
| - Aquifer Overdraft |  |  | Not Applicable |  | - Not Applicable |  |  |
| Insufficient Flows in Water Courses |  |  | Slight Worsening |  | - Increases infiltration and reduces runoff. |  |  |
| - WATER – QUALITY |  |  |  |  |  |  |  |
| - In Groundwater: |  |  |  |  |  |  |  |
| - Harmful Levels of Pesticides |  |  | Neutral |  | - Depending on the type of deep tillage, infiltration and runoff may be decreased or increased |  |  |
| - Excessive Nutrients and Organics |  |  | Slight to Moderate Worsening |  | - Deep ripping increases infiltration resulting in greater leaching potential. |  |  |
| - Excessive Salinity |  |  | Not Applicable |  | - Not Applicable |  |  |
| - Harmful Levels of Heavy Metals |  |  | Not Applicable |  | - Not applicable. |  |  |
| - Harmful Levels of Pathogens |  |  | Not Applicable |  | - Not applicable. |  |  |
| - Harmful Levels of Petroleum |  |  | Not Applicable |  | - Not applicable. |  |  |
| - In Surface Water: |  |  |  |  |  |  |  |
| - Harmful Levels of Pesticides |  |  | Slight to Substantial Improvement |  | - The slight increase in infiltration is negated by the increased erosion of the bare and loosened soil conditions that result. |  |  |
| - Excessive Nutrients and Organics |  |  | Slight Improvement |  | - Increase in infiltration may be negated by the increased erosion of the bare and loosened soil conditions that result from the deep tillage method. |  |  |
| - Excessive Suspended Sediment and Turbidity |  |  | Neutral |  | - Increase in infiltration may be negated by the increased erosion of the bare and loosened soil conditions that result from the deep tillage method. |  |  |
| - Excessive Salinity |  |  | Slight Improvement |  | - Deep ripping increases infiltration, reducing runoff and erosion. |  |  |
| - Harmful Levels of Heavy Metals |  |  | Slight Improvement |  | - Increase in infiltration may be negated by the increased erosion of the bare and loosened soil conditions that result from the deep tillage method. |  |  |
| - Harmful Temperatures |  |  | Not Applicable |  | - Not applicable. |  |  |
| - Harmful Levels of Pathogens |  |  | Neutral |  | - Increase in infiltration may be negated by the increased erosion of the bare and loosened soil conditions that result from the deep tillage method |  |  |
| - Harmful Levels of Petroleum |  |  | Neutral |  | - Increase in infiltration may be negated by the increased erosion of the bare and loosened soil conditions that result from the deep tillage method |  |  |
| - AIR – QUALITY |  |  |  |  |  |  |  |
| - Particulate Matter less than 10 Micrometers in Diameter (PM 10) |  |  | Not Applicable |  | - Not applicable. |  |  |
| Particulate Matter less than 2.5 Micrometers in Diameter (PM 2.5) |  |  | Not Applicable |  | - Not applicable. |  |  |
| Excessive Ozone |  |  | Not Applicable |  | - Not applicable. |  |  |
| Excessive Greenhouse Gas: |  |  |  |  |  |  |  |
| - CO2 (Carbon Dioxide) |  |  | Not Applicable |  | - Not applicable. |  |  |
| - N20 (Nitrous Oxide) |  |  | Not Applicable |  | - Not applicable. |  |  |
| - CH4 (Methane) |  |  | Not Applicable |  | Not applicable. |  |  |
| Ammonia (NH3) |  |  | Not Applicable |  | Not Applicable |  |  |
| Chemical Drift |  |  | Not Applicable |  | Not applicable. |  |  |
| Objectionable Odors |  |  | Not Applicable |  | Not applicable. |  |  |
| Reduced Visibility |  |  | Not Applicable |  | Not applicable. |  |  |
| Undesirable Air Movement |  |  | Not Applicable |  | Not applicable. |  |  |
| Adverse Air Temperature |  |  | Not Applicable |  | Not applicable. |  |  |
| PLANTS – SUITABLIITY |  |  |  |  |  |  |  |
| Plants not Adapted or Suited |  |  | Not Applicable |  | Not applicable. |  |  |
| PLANTS - CONDITION |  |  |  |  |  |  |  |
| Productivity, Health, and Vigor |  |  | Slight to Substantial Improvement |  | Enhanced root growth and the reduction of concentrated contaminants improves plant health and vigor. |  |  |
| Threatened or Endangered Plant Species: |  |  |  |  |  |  |  |
| - Plant Species Listed or Proposed for Listing Under the Endangered Species Act |  |  | Not Applicable |  | Not applicable. |  |  |
| - Declining Species, Species of Concern |  |  | Not Applicable |  | Not applicable. |  |  |
| Noxious and Invasive Plants |  |  | Not Applicable |  | Deep tillage that mixes the surface soil may enhance the growth of noxious and invasive plants. Deep tillage that buries plants and seed deep in the subsoil reduces germination. |  |  |
| Forage Quality and Palatability |  |  | Slight Improvement |  | Deep tillage increases rooting depth and vigor. |  |  |
| Wildfire Hazard |  |  | Not Applicable |  | Not applicable. |  |  |
| ANIMALS - FISH AND WILDLIFE |  |  |  |  |  |  |  |
| Inadequate Food |  |  | Not Applicable |  | Not applicable. |  |  |
| Inadequate Cover/Shelter |  |  | Not Applicable |  | Not applicable. |  |  |
| Inadequate Water |  |  | Not Applicable |  | Not applicable. |  |  |
| Inadequate Space |  |  | Not Applicable |  | Not applicable. |  |  |
| Habitat Fragmentation |  |  | Not Applicable |  | Not applicable. |  |  |
| Imbalance Among and Within Populations |  |  | Not Applicable |  | Not applicable. |  |  |
| Threatened and Endangered Fish and Wildlife Species: |  |  |  |  |  |  |  |
| - Fish and Wildlife Species Listed or Proposed for Listing Under the Endangered Species Act |  |  | Neutral |  | Activities are designed, installed, and mitigated to an extent to maintain or enhance species of concern. |  |  |
| - Declining Species, Species of Concern |  |  | Neutral |  | Activities are designed, installed, and mitigated to an extent to maintain or enhance species of concern. |  |  |
| ANIMALS – DOMESTIC |  |  |  |  |  |  |  |
| Inadequate Quantities and Quality of Feed and Forage |  |  | Slight to Moderate Improvement |  | Forage production may be enhanced by increasing rooting depth and vigor. |  |  |
| Inadequate Shelter |  |  | Not Applicable |  | Not applicable. |  |  |
| Inadequate Stock Water |  |  | Not Applicable |  | Not applicable. |  |  |
| Stress and Mortality |  |  | Not Applicable |  | Not applicable. |  |  |
| HUMAN – ECONOMICS |  |  |  |  |  |  |  |
| Land - Change in Land Use |  |  | Not applicable. |  | Not applicable. |  |  |
| Land – Land in Production |  |  | Not applicable |  | Not applicable. |  |  |
| Capital – Change in Equipment |  |  | Moderate increase. |  |  |  |  |
| Capital - Total Investment Cost |  |  | Not applicable. |  |  |  |  |
| Capital – Annual Cost |  |  | Slight to moderate increase. |  |  |  |  |
| Capital – Credit and Farm Program Eligibility |  |  | Situational. |  |  |  |  |
| Labor - Labor |  |  | Slight to moderate increase |  | Slight to moderate increase for performing tillage operations. |  |  |
| Labor – Change in Management Level |  |  | Negligible |  |  |  |  |
| Risk - Yield |  |  | Slight Decrease |  | Slight decrease due to improved infiltration and root penetration. |  |  |
| Risk - Flexibility |  |  | Slight Decrease |  | Slight decrease due to more conductive growing conditions. |  |  |
| Risk - Timing |  |  | Moderate Increase |  | Moderate increase - practice should be implemented prior to planting. |  |  |
| Risk – Cash Flow |  |  | Slight Increase |  | Negligible to slight increase due to trips over the field. |  |  |
| Profitability – Change in Profitability |  |  | Slight to moderate increase. |  |  |  |  |
| HUMAN - CULTURAL |  |  |  |  |  |  |  |
| Cultural Resources and/or Historic Properties Present or Suspected to be PRESENT |  |  | Slight to Substantial Increase |  | Historic properties below existing plow zone can be adversely effected. |  |  |
| HUMAN – ENERGY |  |  |  |  |  |  |  |
| Depletion of Fossil Fuel Resources |  |  | Slight to Moderate Increase |  | The amount of energy expended depends on depth and frequency of tillage |  |  |
| Underutilization of Non-Fossil Energy Resources |  |  | Not Applicable |  | Not applicable. |  |  |

Human Considerations Explanation

| Considerations | Physical effects indicate: |
| --- | --- |
| Land - Change in Land Use | The degree to which implementing the conservation practice is expected to cause a change from one land use to another. |
| Land - Land in Production | The degree to which implementing the conservation practice is expected to cause an increase or decrease in the amount of land in production. |
| Capital - Change in Equipment | The degree to which implementing the conservation practice is expected to cause an increase or decrease in the amount of capital equipment required for farm or ranch operations. |
| Capital - Total Investment Cost | A qualitative measure of the increase in total investment dollars required in order to implement the conservation practice. |
| Capital - Annual Cost | A qualitative measure of the expected change in annual capital costs required in order to operate and maintain the conservation practice. |
| Capital - Credit & Farm Program Eligibility | Included to make conservation planners aware of the potential availability of funding for implementing conservation practices. |
| Labor – Labor | The degree to which implementing the conservation practice is likely to cause an increase or decrease in the total amount of overall farm or ranch labor required for operations. |
| Labor - Change in Management Level | The degree to which implementing the conservation practice is likely to cause an increase or decrease in the total amount of required active management on a farm or ranch. |
| Risk – Yield | The degree to which risk, as related to crop or livestock yields, is expected to increase or decrease as a result of implementing the conservation practice. |
| Risk – Flexibility | The degree to which risk, as related to the flexibility of farm or ranch operations, is expected to increase or decrease as a result of implementing the conservation practice. For example, converting from flood irrigation to a sprinkler system gives a farmer an increase in flexibility of irrigation, which results in a decrease in the level of risk associated with inflexibility of operations. |
| Risk – Timing | The degree to which risk, as related to the timing of farm or ranch operations, is expected to increase or decrease as a result of implementing the conservation practice. |
| Risk - Cash Flow | The degree to which risk, as related to cash flow in farm or ranch operations, is expected to increase or decrease as a result of implementing the conservation practice. |
| Profitability - Change in Profitability | The degree to which farm or ranch profitability is expected to increase or decrease as a result of implementing the conservation practice. |
| Cultural Resources and/or Historic Properties Present or Suspected to be Present | The degree to which implementation of the conservation practice is expected to increase or decrease the risk of cultural resource disturbance, degradation, or loss. |
| Depletion of Fossil Fuel Resources | Inefficient use of fossil-originated energy sources (diesel, gasoline, propane, natural gas, coal), lubricants, and other materials. |
| Underutilization of Non-Fossil Energy Sources | Available and cost-effective alternative energy sources (solar, wind, biofuel, hydroelectric, geothermal) are not being used or are being used inefficiently. |