CONSERVATION TREATMENT EFFECTS

Level I Analysis

Oregon Farm

Conservation Management Unit  – Dairy Farm, Irrigated Cropland, Manure Application, 160 acres

Crop Rotation: 2 Years: Corn Silage, 2 Years: Grass/Legume Silage.

| CONSERVATION TREATMENT: RMS#1 - Intensified crop rotation, improved waste application and fertilizer management. Filter strips at field edges and streambank protection. | RESOURCE PROBLEMS: Surface water quality from nutrients and animal wastes. Potential N leaching below root zone. Low yields and potential grass tetany problem from excess K. Streambank erosion. |
| --- | --- |
| POSITIVE EFFECTS + | NEGATIVE EFFECTS - |
| + Domestic animal feed requirements improved due to decrease in potential grass tetany problem + Less labor required, less time spent hauling and spreading dry manure + Plant yields increase for corn and grass/legume silage + Nutrient management improves plant management and yields + Streambank erosion is reduced + Aquatic habitat for fish and wildlife is improved + Potential for excess nutrients leaching to groundwater is reduced + Commercial fertilizer (N,P) use is reduced + Surface loss of nutrients, organics and pathogens is reduced | - Increased cost, time and labor for soil tests and waste analyses - Increased labor and machinery time during February, March and April to haul and spread dry manure - Upgrade to larger capacity manure spreader - Install filter strips - Lost crop production to filter strip acres - Install streambank protection - Operation, maintenance and repair costs for installed improvements |

CONSERVATION TREATMENT EFFECTS

# Level II Analysis

Oregon Farm

Conservation Management Unit  – Dairy Farm, Irrigated Cropland, Manure Application, 160 acres

Crop Rotation: 2 Years: Corn Silage, 2 Years: Grass/Legume Silage.

| CONSERVATION TREATMENT: RMS#1 – Intensified crop rotation, improved waste application and fertilizer mgt. Filter strips at field edges and streambank protection. | RESOURCE PROBLEMS: Surface water quality from nutrients and animal wastes. Potential N leaching below root zone. Low yields and potential grass tetany problem from excess K. Streambank erosion. |
| --- | --- |
| POSITIVE EFFECTS + | NEGATIVE EFFECTS - |
| + Fertilizer (K) reduced by 87Lbs/Ac results in less grass tetany problem + Labor/Equipment time reduced 40 hours annually to haul/spread dry manure + Plant production increases: .50 Ton/Ac Corn Silage .25 Ton/Ac Grass/Legume Silage + Streambank erosion reduced 36 Tons/Yr + N and P fertilizer application reduced N 60Lbs/Ac/Yr P 17Lbs/Ac/Yr + Reduced cost of fertilizer - $39/Ac + Cost Share Payment Wastes/Nutrient Management - $5/Ac Filter Strips - $300 Streambank Protection - $2,500 | - Cost of soil/manure testing and management - $7/Ac/Yr - Upgrade to larger capacity manure spreader - $7,000 - Increased cost for dry manure application $2/Ac/Yr - N fertilizer cost on grass/legume silage $6/Ac - 2 acres lost crop production to install filter strips - Install 2 acres filter strips - $400 Annual O&M - $50/Yr - Install streambank protection - $4,500 Annual O&M - $150/Yr |

CONSERVATION TREATMENT EFFECTS

# Level III Analysis

Oregon Farm

Conservation Management Unit  – Dairy Farm, Irrigated Cropland, Manure Application, 160 acres

Crop Rotation: 2 Years: Corn Silage, 2 Years: Grass/Legume Silage.

| CONSERVATION TREATMENT: RMS#1 - Intensified crop rotation, improved waste application and fertilizer mgt. Filter strips at field edges and streambank protection. | RESOURCE PROBLEMS: Surface water quality from nutrients and animal wastes. Potential N leaching below root zone. Low yields and potential grass tetany problem from excess K. Streambank erosion. |
| --- | --- |
| POSITIVE EFFECTS + | NEGATIVE EFFECTS - |
| + Excess fertilizer (K) reduced, less grass tetany 87Lbs/Ac/Yr * $.16/Lb * 160Acres = $2,227/Yr + Less Grass Tetany, Lower Veterinary Costs $500/Yr + Labor/Equipment reduced to haul/spread manure 120 Hours/Year * $15/Hour = $1,800/Yr + Plant production increases: Corn Silage: .50 T/Ac/Yr * 160 Ac * $50/Ton = $4,000/Yr Grass/Legume Silage: .25 T/Ac/Yr * 160 Ac * $40/Ton = $1,600/Yr + Streambank Erosion Reduced 36 Tons/Yr + Fertilizer Application Reduced: N 60 Lbs/Ac/Yr * 160Ac = 9,600Lbs/Yr P 17 Lbs/Ac/Yr * 160Ac = 2,720Lbs/Yr + Reduced cost of fertilizer Application $39/Ac * 160Ac = $6,240/Yr + Cost Share Payment Wastes/Nutrient Management = $5/Ac/Yr * 160 = $800 (5Yrs) Filter Strips = $300 Streambank Protection = $2,500 Installation Benefit = $2,800 Amortized Benefit 8%, 20 Yr = $285/Yr Annual Benefits = $17,167/Yr $17,452/Yr | - Cost of soil/manure testing and management $7/Ac/Yr * 160 Acres = $1,120/Yr - Upgrade to larger capacity manure spreader = $7,000 - Increased cost for dry manure application $2/Ac/Yr * 160 Acres = $320/Yr - N fertilizer cost on grass/legume silage $6/Ac * 160 Acres = $960/Yr - Lost crop production to install filter strips 2 Acres * $200/Ac/Yr Income = $400/Yr - Install 2 acres filter strips - $400 Annual O&M = $50/Yr - Install streambank protection = $4,500 Annual O&M = $150/Yr Installation Cost = $11,900 Amortized Cost 8%, 20 Yr = $1,212/Yr Annual Cost = $3,000/Yr $4,212/Yr |