## Minimum Standard (Legend, Mapunit, Data Mapunit)

The NASIS data elements that make up a defined minimum standard to support current national programs are shown in the table below. The standard includes data elements from National Bulletin 430-5-7 (SOI Soil Information in Field Office Technical Guides) and other data elements that support current national interpretations, required national NASIS calculations (per NB 430-14-3 SOI-Calculated Soil Interpretation Factors in NASIS), and the Conservation Assessment Ranking Tool (CART).

Data integrity check reports are provided to ensure data comply to this minimum standard.  These reports identify potential errors, omissions, and inconsistencies in NASIS database population. The reports are accessed through the NASIS Data Integrity Report Site. An individual report will assess the data integrity of one or a few interrelated data elements, therefore many reports will be developed but only a select few are currently available. The available reports are marked “Ready to Use” in NASIS and listed in the last three columns in the table below for the data element and type of check in which they apply. The number of reports will increase with time as new reports become available. For more details, refer to the data integrity report named “Read First About Data Integrity Reports.”

Application: The primary intent is to apply this minimum standard to official data that already exist in NASIS. Any new mapunits or new data mapunits should meet the population requirements of a full standard, such as NI430-305 Exhibit A, or a similar full standard.

Minimum Standard. Build Date: December 2018

| NASIS Table | Data Element | Data Population Guidance UNDER DEVELOPMENT | Population sequence | NASIS Table ACRONYM | CHECK Report - Empty Table | CHECK Report - NULL | CHECK Report - Data Quality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mapunit | Mapunit NASIS Site | A Mapunit and its Rep DMU should be in the same NASIS Site | 01 | M |  |  | 01aM-Mapunits with Data Mapunits in Different NASIS Site |
| Correlation | Representative DMU | The DMU that is representative of the map unit is checked as the Rep DMU. Only one DMU is checked as Rep. | 01 | COR | 01COR-RV DMU NOT Indicated, Is Missing, or Has Multiple RVs |  | 01COR-RV DMU NOT Indicated, Is Missing, or Has Multiple RVs |
| Data Mapunit | DMU NASIS Site | A Mapunit and its Rep DMU should be in the same NASIS Site | 01 | D |  |  |  |
| Data Mapunit | DMU Description | Under Development | 01 | D | Under Development |  |  |
| Data Mapunit | Order of Mapping |  | 01 | D |  |  |  |
| Component | All | Find DMUs with no or empty Component Table | 01 | D | 01Ca-Component Table is Missing |  |  |
| Horizon | All | Find Components with Taxon of series, taxadjunct, family, taxon above family, or variant (obsolete) that have no, or empty, Horizon Table | 01 | H | 01Cb-Horizon Table is Missing |  |  |
| Component | Component Name |  | 02 | C |  | 02Cb-Compname Contains Local Phase or is NULL | 02Cc-Major Compname NOT Consistent with Mapunit Name |
| Component | Taxon Kind |  | 03 | C |  | 03Ca-Component taxon kind is NULL | 03C-Component Taxon Kind Equals Series and No OSD; 03C-Miscellaneous Area Names Not Approved |
| Component | Major Component Flag |  | 04 | C |  |  | 04Ca-DMU with No Major Component; 04Cb-Majorcompflag NOT Indicated or is Minor Comp |
| Component | Comp % (RV) |  | 05 | C |  | 05Ca-RV Component Percent is NULL or Zero | 05Cb-RV Component Percent NOT Equal 100; 05Cc-Identical Major RV Component Percents in Same DMU |
| Component | Local Phase | Used only when needed to distinguish various component phase criteria | 05 | C |  |  |  |
| Component | MAAT (L,RV,H) |  | 06 | C |  |  |  |
| Component | MAP (L,RV,H) |  | 06 | C |  |  |  |
| Component | Frost Free Days (L,RV,H) |  | 06 | C |  |  |  |
| Component | Slope Gradient (L,RV,H) |  | 06 | C |  |  |  |
| Component | Erosion Class |  | 06 | C |  |  |  |
| Component | Drainage Class |  | 06 | C |  |  |  |
| Component | Order |  | 07 | C |  |  |  |
| Component | Suborder |  | 08 | C |  |  |  |
| Component | Great Group |  | 09 | C |  |  |  |
| Component | Subgroup |  | 10 | C |  |  |  |
| Component | Particle Size |  | 11 | C |  |  |  |
| Component | CEC Activity Class |  | 11 | C |  |  |  |
| Component | Reaction |  | 11 | C |  |  |  |
| Component | Temp Class |  | 11 | C |  |  |  |
| Component | Temp Regime |  | 11 | C |  |  |  |
| Component | HAHT Material Class | Populated only for certain subgroups (those listed on pp. 323-324 KST 12 Ed.) or soils with HAHT from soil surface to 50 cm or to a root limiting layer, whichever is shallower; otherwise is NULL. | 11 | C |  |  |  |
| Component Taxonomic Family Mineralogy | Mineralogy |  | 12 | CTFM |  |  |  |
| Component Taxonomic Moisture Class | Moisture Class |  | 12 | CTMS |  |  |  |
| Component Tax Family Other | Family Other |  | 12 | CTFO |  |  |  |
| Component | Keys to Taxonomy Edition |  | 13 | C |  |  |  |
| Component | Taxonomic Classification |  | 14 | C |  |  |  |
| Component Geomorphic Description | Representative?, Feature Type, Feature Name, Feature Id and Exists on Feature ID for a Landscape row and a Landform row |  | 15 | CGD |  |  |  |
| Component 3D Surface Morphometry | Hills, Terraces, Mountains, or Flats as applicable (for the Landform Feature) |  | 16 | C3D |  |  |  |
| Component 2D Surface Morphometry | Hillslope Profile (for the Landform Feature) |  | 16 | C2D |  |  |  |
| Component Slope Shape Morphometry | Across and Up/Down (for the Landform Feature) |  | 16 | CSS |  |  |  |
| Component Parent Material Group | Parent Material Group Name |  | 17 | CPMG |  |  |  |
| Component Parent Material | Kind, Origin |  | 18 | CPM |  |  |  |
| Component Parent Material Group | Representative |  | 19 | CPMG |  |  |  |
| Component Month | Month |  | 20 | CM |  |  |  |
| Component Month | 12 Months Flooding Frequency and Duration |  | 21 | CM |  |  |  |
| Component Month | 12 Months Ponding Frequency, Duration and Depth (L,RV,H) |  | 21 | CM |  |  |  |
| Component Soil Moisture | Moisture Status | Required if status = wet. | 22 | CSM |  |  |  |
| Component Soil Moisture | Top Depth (L,RV,H) |  | 22 | CSM |  |  |  |
| Component Surface Fragments | Cover % and Size (L,RV,H) |  | 23 | CSF |  |  |  |
| Component Restrictions | Kind, Top Depth (L,RV,H), Hardness |  | 24 | CR |  |  |  |
| Component | Albedo Dry (RV) |  | 25 | C |  |  |  |
| Component | Aspect (RV), Clockwise, Counter Clockwise |  | 25 | C |  |  |  |
| Component | Hydric Rating |  | 25 | C |  |  |  |
| Component Hydric Criteria | Hydric Criterion |  | 26 | CHC |  |  |  |
| Horizon | Top Depth (RV) |  | 27 | H |  |  |  |
| Horizon | Bottom Depth (RV) |  | 27 | H |  |  |  |
| Horizon | Thickness (RV) |  | 28 | H |  |  |  |
| Horizon | Disc, Master, Prime, Sub |  | 29 | H |  |  |  |
| Horizon | Designation Suffix |  | 29 | HDS |  |  |  |
| Horizon | Designation | H1, H2, etc. are acceptable. However, organic horizons must have Master populated as O, plus suffix then Horizon Designation calculated; and bedrock layers must be populated as Cr or R, with appropriate Master and suffix populated then Horizon Designation calculated. | 30 | H |  |  |  |
| Horizon | Total Sand (RV) |  | 31 | H |  |  |  |
| Horizon | Sand Fractions (RV) |  | 32 | H |  |  |  |
| Horizon | Total Clay (L,RV,H) |  | 33 | H |  |  |  |
| Horizon | Total Silt (RV) |  | 34 | H |  |  |  |
| Horizon Texture | Texture or In Lieu Term |  | 35 | HT |  |  | 01cc-35HTa-Obsolete Lieutex; 35HTb-Texture Class and Lieutex in Same Row; 35HTc-Multiple Tex Classes/Lieutex Terms and NOT Stratified; 35HTd-RV Texture Class not Consistent with RV Particle Size |
| Horizon Texture Modifier | Horizon Texture Modifier |  | 36 | HTM |  |  |  |
| Horizon Texture Group | Tex Mod & Class & Representative |  | 37 | HTG | 37HTGa-Texture Group Table is Missing | 37HTGc-Texture Group NOT Calculated or is NULL | 37HTGb-RV Texture NOT Indicated or Multiple RVs Indicated; 37HTGd-Stratified Single Textures |
| Horizon Fragments | Vol % and Size (L,RV,H), kind, shape, and hardness |  | 38 | HF |  |  |  |
| Horizon | Organic Matter (L,RV,H) |  | 39 | H |  |  |  |
| Horizon | Bulk Density 0.33 bar (L,RV,H) |  | 40 | H |  |  |  |
| Horizon | Rock fragments >10 inches and 3-10 inches (L,RV,H) |  | 41 | H |  |  |  |
| Horizon | Sieves #4, #10, #40, #200 (L,RV,H) |  | 42 | H |  |  |  |
| Horizon | pH H2O (L,RV,H) | The Data Integrity Check reports focus on mineral soils | 43 | H |  |  |  |
| Horizon | pH CaCl2 (L,RV,H) | The Data Integrity Check reports focus on organic layers | 44 | H |  |  |  |
| Horizon | Gypsum (L,RV,H) |  | 45 | H |  |  |  |
| Horizon | CaCO3 (L,RV,H) |  | 46 | H |  |  |  |
| Horizon | SAR (L,RV,H) |  | 47 | H |  |  |  |
| Horizon | ESP (L,RV,H) |  | 48 | H |  |  |  |
| Horizon | EC (L,RV,H) |  | 49 | H |  |  |  |
| Horizon | Ksat (L,RV,H) |  | 50 | H |  |  |  |
| Horizon Structure Group | Structure |  | 51 | HSGR |  |  |  |
| Horizon Structure | Grade, Size, Type |  | 52 | HS |  |  |  |
| Horizon Structure | Structure ID and Parts to Structure ID where applicable |  | 52 | HS |  |  |  |
| Horizon Structure Group | Representative |  | 53 | HSGR |  |  |  |
| Horizon | CEC or ECEC (L,RV,H) |  | 54 | H |  |  |  |
| Horizon | LEP (L,RV,H) |  | 55 | H |  |  |  |
| Horizon | Db oven dry (RV) |  | 56 | H |  |  |  |
| Horizon | Water Contents, 0.33, 0.10 (for sands), 15 Bar, Satiated (RV) |  | 57 | H |  |  |  |
| Horizon | LL and PI (L,RV,H) |  | 58 | H |  |  |  |
| Horizon | Kw, Kf |  | 59 | H |  |  |  |
| Horizon | Sum of Bases (L,RV,H) |  | 60 | H |  |  |  |
| Horizon | AWC (L,RV,H) |  | 61 | H |  |  |  |
| Horizon AASHTO | AASHTO (RV) |  | 62 | HA |  |  |  |
| Horizon | AASHTO Group Index (L,RV,H) |  | 63 | H |  |  |  |
| Horizon Unified | Unified (RV) |  | 64 | HU |  |  |  |
| Component | Cover Kind 1 |  | 65 | C |  |  |  |
| Component | HSG (Hydrologic Soil Group) |  | 66 | C |  |  |  |
| Component | Concrete Corrosion |  | 66 | C |  |  |  |
| Component | Steel Corrosion |  | 66 | C |  |  |  |
| Component | T Factor |  | 66 | C |  |  |  |
| Component | WEG/WEI |  | 66 | C |  |  |  |
| Component | Frost Action |  | 66 | C |  |  |  |
| Component | Non-irrigated Land Capability Class |  | 67 | C |  |  |  |
| Component | Non-irrigated Land Capability Subclass |  | 67 | C |  |  |  |
| Component | Irrigated Land Capability Class | Populated for components suited to irrigation; otherwise, NULL. | 67 | C |  |  |  |
| Component | Irrigated Land Capability Subclass | Populated for components suited to irrigation; otherwise, NULL. | 67 | C |  |  |  |
| Component Text | Generated Brief Soil Description |  | 69 | C |  |  |  |
| Component | Range Productivity (If applicable) |  | 69 | C |  |  |  |
| Component Ecological site | Ecological Site (if available) |  | 69 | CES |  |  |  |
| Component | Check Remaining Child Tables |  | 70 | C |  |  |  |
| Mapunit | Mapunit Name |  | 71 | M |  |  |  |
| Mapunit | Kind |  | 72 | M |  |  |  |
| Legend | Area Symbol |  | 74 | L |  |  |  |
| Legend | Area Name |  | 74 | L |  |  |  |
| Legend | Geographic Applicability |  | 75 | L |  |  |  |
| Legend | Project Scale |  | 76 | L |  |  |  |
| Legend | MLRA Office |  | 76 | L |  |  |  |
| Legend Mapunit | Mapunit name |  | 77 | LM |  |  |  |
| Legend Mapunit | Mapunit Symbol |  | 78 | LM |  |  |  |
| Legend Mapunit | Status |  | 79 | LM |  |  |  |
| Legend Mapunit | Total Acres |  | 80 | LM |  |  |  |
| Legend Mapunit | Farm Class |  | 81 | LM |  |  |  |
| Legend Area Overlap | Overlap Area Symbol | Required Area Types=MLRA, State, County. | 82 | LAO |  |  |  |
| Legend Mapunit Area Overlap | Mapunit Symbol |  | 83 | LMAO |  |  |  |
| Data Mapunit Certification History | Certification Date, Certification Kind, Certification Status, Certification Text | Certification Date, Certification Kind=quality control, Certification Status=certified, all components or certified, major components, Certification Text=Statement indicating data population errors have been resolved and the DMU meets this Minimum Standard | 84 | DCH |  |  |  |